# PredictX Technical Specification: Turbofan Maintenance & Inspection Standards

# 1. High Pressure Compressor (HPC) Maintenance and Borescope Standards

The High Pressure Compressor assembly comprises stages 1 through 9. Degradation is accelerated by particulate ingestion, thermal cycling, and blade tip aerodynamic erosion.

Inspection Protocol (Borescope):
1. Insert flexible 4mm articulating video borescope through access ports CP-01 (stages 1-3), CP-02 (stages 4-6), and CP-03 (stages 7-9).
2. Inspect leading edges for foreign object damage (FOD). Maximum allowable nick depth is 0.35 mm on rotor blades.
3. Inspect blade tip clearances using calibrated visual gauge. Clearances exceeding 1.25 mm require compressor shroud segment replacement.
4. If telemetry indicates static pressure s_11 (Ps30) drop exceeding 1.5% below baseline, perform immediate compressor wash.

Compressor Wash Procedure:
- Utilize approved aqueous demineralized surfactant solution heated to 60°C (140°F).
- Motor engine on starter at cranking speed (approx. 18-22% Nc) with fuel and ignition deactivated.
- Inject wash solution through manifold for 4 minutes, followed by a 10-minute soak, and a 6-minute clean-water rinse.
- Dry run engine at idle for 15 minutes before returning to active operational service.

# 2. Turbine Hot Section and Exhaust Gas Temperature (EGT) Management

The High Pressure Turbine (HPT) single-stage blade set operates under extreme temperatures exceeding 1600 R.

EGT (s_4 / T50) Divergence Action Matrix:
- Continuous EGT > 1415 R: Flag for thermal efficiency audit.
- Continuous EGT > 1425 R: High risk alarm. Limit maximum power setting to 88% TRA.
- Peak Transient EGT > 1445 R: Automatic work order creation. Ground machine for borescope inspection of HPT nozzle guide vanes and cooling film hole blockage.

Thermal Barrier Coating (TBC) Loss Criteria:
Loss of yttria-stabilized zirconia (YSZ) coating over greater than 15% of blade suction side surface mandates immediate rotor module replacement to prevent base nickel-alloy melting.

# 3. Lubrication, Bearing Chambers, and Chip Detection

The engine oil subsystem lubricates bearings #1 through #5.

Magnetic Chip Detectors (MCD):
- Visual inspection required every 50 operating cycles.
- Class A debris: Fine metallic sludge (< 0.1 mm) — Clean plug and resume service.
- Class B debris: Flakes (0.1 mm - 0.5 mm) — Flush oil system, resample at 10-cycle interval.
- Class C debris: Chunks, roller slivers, or bearing cage fragments (> 0.5 mm) — Immediate grounding. Teardown bearing chamber 2 and 3.

Oil Filter Differential Pressure (Delta-P):
Normal delta-P: 3 to 8 psid. If delta-P indicator pop-up button extends (> 15 psid), the filter bypass valve has opened. Replace filter element within 5 operating cycles.